PCB Report – Board 1

555 Timer as Astable Vibrator

Objective:

A 555 timer can be used in modes such as Monostable, Bistable, and Astable Multivibrator. Here in this lab, the 555 timer is used as an Astable Multivibrator to output a signal with a certain frequency and duty cycle. Also, comparing the rise time, fall time, and amplitude of voltage output from a slow 555 timer NE555 used.

Plan of Record:

Features of board:

- 5V power input rating.

- Slow 555 timer IC(NE555).

- Designed for Duty cycle of 66% and 500Hz.

- Different loads attached 50, 300,1k and 10k to limit current.

Risk Reduction:

- Added LEDs to indicate power supply is working fine.

- Separated the modules as power, 555timer and load using headers/switches.

- Proper labelling for each input and outputs

- Added decoupling capacitor to avoid switching noise in power rail.

- Plane is poured with copper to make it GND to avoid mutual loop inductance due to large loop area.

What does it mean to work?

- Read 5V from power input test point.

- Read 555 timer output signal with nearly 66% duty cycle and 500Hz frequency.

- Observe dimness in load LEDs according to increase in load resistor values.

- Draw current less than 200mA that NE555 supports.

- Rist time and fall time closest to that mentioned in Datasheet.

Components listing:

- Resistors: 1K, 50, 500,10K

- Capacitors: 22uF, 1uF

- Power jack

- Headers

- LEDS

Schematic:

Layout:

Board:

Assembled board with LEDS lit up.

Calculation:

Here, I have calculated the resistor and capacitor values for a frequency of 500Hz and a duty cycle of Ton as 66.67%.

T = Ton + Toff

Ton = 0.693(R1+R2) C

Toff = 0.693(R2) C

T = 0.693(R1 + 2R2) C

F = 1/T = 1.44/ (R1 + 2R2) C

The components provided are 1K and 1uF so, considering R1=R2= 1K and C = 1uF

F= 1.44/(3K) 1uF = 480Hz

T= 1/480 = 2.08ms

Ton = 0.693(1K + 1K) 1uF = 1.386ms

Duty cycle = Ton/T = 66.6%

Scope output:

- 5V from power supply

- Figure of merits from 555 timer output

Noted signal output with amplitude 4.7V, frequency of 489.61Hz and 66.04% duty cycle which is nearest to the design.

- Rise time of 555 timer output without load

Rise time of 68.2ns observed due to use of slow 555 timer output.

- Fall time of 555 timer output without load

Fall time of 27ns indicates the fast turning on of the NPN transistor.

- Rise time with load connected.

Noted rise time of 53.2 which is less compared to without load due to reduce in amplitude of output due to load.

- Fall time with the load connected.

Observed fall time of 51.4ns which is higher than without load due to slow turning of NPN transistor.

- Voltage across 50ohm resistor

Voltage Amplitude of 1.6V read across 50ohm resistor and current drawn = V/R = 1.6/50 = 32mA.

- Voltage across 300ohm resistor

Voltage Amplitude of 1.8V read across 300ohm resistor and current drawn = V/R = 1.8/300 = 6mA

- Voltage across 1K resistor

Voltage Amplitude of 1.8V read across 300ohm resistor and current drawn = V/R = 1.8/1K = 1.8mA.

- Voltage across 10K resistor

Voltage Amplitude of 2V read across 10K resistor and current drawn = V/R = 2/10K = 0.2mA.

- Thevenin Voltage

The voltage measured without load is 4.7V which is equal to Thevenin voltage, and the load voltage can be 3.8V from point 5. The Thevenin resistance RL = Rth (Vth – VL)/VL = 40.9(4.7-3.8)/3.8 = 9.7ohm.

What all worked:

| Parameters | Result | Remarks |
| --- | --- | --- |
| Read 5V from power input test point. | Worked |  |
| Read 555 timer output signal with nearly 66% duty cycle and 500Hz frequency. | Worked |  |
| Observe dimness in load LEDs according to increase in load resistor values. | Worked |  |
| Draw current less than 200mA that NE555 supports. | Worked |  |
| Rist time and fall time closest to that mentioned in Datasheet. | Worked |  |

Mistakes:
No errors seen in the board.

Key learnings:

- Switching noise depends on how fast IC switches.

- Benefit of adding decoupling capacitor to compensate sudden surge in current and avoid switching noise.

- How to use Altium software to create schematics, layout and BoM.

- Design board in modules so that can help in bring up and debugging.

- Importance of risk reduction in project development.